Q1)

A main code and two subroutines, GPIO initialization & Signal Send, are provided in the following codes:

GPIO Initialization

GPIO_PORTB_DATA 	EQU 0x400053FC

GPIO_PORTB_DIR 		EQU 0x40005400

GPIO_PORTB_AFSEL	EQU 0x40005420

GPIO_PORTB_DEN 		EQU 0x4000551C

GPIO_PORTB_PUR 		EQU 0x40005510

GPIO_PORTB_PDR 		EQU 0x40005514

IOB 				EQU 0x0F

SYSCTL_RCGCGPIO 	EQU 0x400FE608

;***************************************************************

; Program section

;***************************************************************

;LABEL		DIRECTIVE	VALUE		COMMENT

;LABEL DIRECTIVE VALUE COMMENT

AREA rutins , CODE, READONLY

THUMB

EXPORT InitGPIO ;

InitGPIO	PROC

LDR R1 , =SYSCTL_RCGCGPIO

LDR R0 , [ R1 ]

ORR R0 , R0 , #0x2;Port B clock enabled

STR R0 , [ R1 ]

NOP		;Wait for clock to stabilize

NOP

NOP

LDR R1 , =GPIO_PORTB_DIR ; Config of Port B starts

LDR R0 , [ R1 ]

BIC R0 , #0xFF

ORR R0 , #IOB;00001111 1->output

STR R0 , [ R1 ]

LDR R1 , =GPIO_PORTB_AFSEL

LDR R0 , [ R1 ]

BIC R0 , #0xFF

STR R0 , [ R1 ]

LDR R1 , =GPIO_PORTB_DEN

LDR R0 , [ R1 ]

ORR R0 , #0xFF

STR R0 , [ R1 ]

LDR R1 , =GPIO_PORTB_PUR

LDR R0 , [ R1 ]

ORR R0 , #0xF0

STR R0 , [ R1 ]

BX LR; end

;***************************************************************

; End of the program  section

;***************************************************************

;LABEL      DIRECTIVE       VALUE                           COMMENT

ALIGN

ENDP

END

Signal Send

GPIO_PORTB_DATA 	EQU 0x400053FC

;***************************************************************

; Program section

;***************************************************************

;LABEL		DIRECTIVE	VALUE		COMMENT

;LABEL DIRECTIVE VALUE COMMENT

AREA rutins , CODE, READONLY

THUMB

EXPORT SignalSend ;

SignalSend	PROC

ORR		R0, R0, #0xF0 ; Data is set

LDR		R1,=GPIO_PORTB_DATA; Data address in R1

STR		R0,[R1];	Corresponding Outputs set high

BX LR; end

;***************************************************************

; End of the program  section

;***************************************************************

;LABEL      DIRECTIVE       VALUE                           COMMENT

ALIGN

ENDP

END

Main Code

;***************************************************************

; Program_Directives.s

; Copies the table from one location

; to another memory location.

; Directives and Addressing modes are

; explained with this program.

;***************************************************************

;***************************************************************

; EQU Directives

; These directives do not allocate memory

;***************************************************************

;LABEL		DIRECTIVE	VALUE		COMMENT

OFFSET  	EQU     	0x10

FIRST	   	EQU	    	0x20000480

STORE		EQU			0x20000410

;***************************************************************

; Directives - This Data Section is part of the code

; It is in the read only section  so values cannot be changed.

;***************************************************************

;LABEL		DIRECTIVE	VALUE		COMMENT

AREA        sdata, DATA, READONLY

THUMB

CTR1    	DCB     	0x10

MSG     	DCB     	"Copying table..."

DCB			0x0D

DCB			0x04

;***************************************************************

; Program section

;***************************************************************

;LABEL		DIRECTIVE	VALUE		COMMENT

AREA    	main, READONLY, CODE

THUMB

EXTERN		OutStr	; Reference external subroutine

EXTERN		InChar; Serial input Added

EXTERN		SignalSend;	GPIO signal send

EXTERN		InitGPIO; GPIO initialize

EXPORT  	__main	; Make available

__main

BL		InitGPIO; GPIO initialized

Begin		MOV 	R0, #0x0001; Set for Full Step config

BL		SignalSend;

B		Begin;

;***************************************************************

; End of the program  section

;***************************************************************

;LABEL      DIRECTIVE       VALUE                           COMMENT

ALIGN

END

Q2)

Q3)

A main code with an additional subroutines, InputCheck, are provided.

InputCheck

GPIO_PORTB_DATA 	EQU 0x400053FC

;***************************************************************

; Program section

;***************************************************************

;LABEL		DIRECTIVE	VALUE		COMMENT

;LABEL DIRECTIVE VALUE COMMENT

AREA rutins , CODE, READONLY

THUMB

EXPORT CheckInput ;

EXTERN delay;

CheckInput	PROC

LDR		R1,=GPIO_PORTB_DATA; Data address in R1

MOV		R2,#0; R2 holds the information cw or ccw

Check		LDR 	R0,[R1];Checks for any input

LSR		R0,#4;

LSRS	R0,#1;

BCC		Delay100

LSRS	R0,#1;

BCC		Delay100

;LSRS	R0,#1;

;BCC		Delay100

;LSRS	R0,#1;

;BCC		Delay100

B		Check

Delay100	MOV32	R0,#1600000;If any input is detected

PUSH{LR}

BL		delay

POP{LR}

LDR 	R0,[R1];	Check Again

LSR		R0,#4;

LSRS	R0,#1;

MOVCC	R2,#1; 1 is the cw direction PB4 pressed

BCC		Released ; If input is detected again wait for relase

LSRS	R0,#1;

MOVCC	R2,#2; 2 is the ccw direction PB5 pressed

BCC		Released

BX LR; if no signal

Released	LDR 	R0,[R1];	It checks for if the switch is open again

LSR		R0,#4;

LSRS	R0,#1;

BCC		Released;	If it is not open

LSRS	R0,#1;

BCC		Released;

BX LR; end

;***************************************************************

; End of the program  section

;***************************************************************

;LABEL      DIRECTIVE       VALUE                           COMMENT

ALIGN

ENDP

END

Main Code

;***************************************************************

; Program_Directives.s

; Copies the table from one location

; to another memory location.

; Directives and Addressing modes are

; explained with this program.

;***************************************************************

;***************************************************************

; EQU Directives

; These directives do not allocate memory

;***************************************************************

;LABEL		DIRECTIVE	VALUE		COMMENT

OFFSET  	EQU     	0x10

FIRST	   	EQU	    	0x20000480

STORE		EQU			0x20000410

;***************************************************************

; Directives - This Data Section is part of the code

; It is in the read only section  so values cannot be changed.

;***************************************************************

;LABEL		DIRECTIVE	VALUE		COMMENT

AREA        sdata, DATA, READONLY

THUMB

CTR1    	DCB     	0x10

MSG     	DCB     	"Copying table..."

DCB			0x0D

DCB			0x04

;***************************************************************

; Program section

;***************************************************************

;LABEL		DIRECTIVE	VALUE		COMMENT

AREA    	main, READONLY, CODE

THUMB

EXTERN		OutStr	; Reference external subroutine

EXTERN		InChar; Serial input Added

EXTERN		SignalSend;	GPIO signal send

EXTERN		InitGPIO; GPIO initialize

EXTERN		delay; Delay is available

EXTERN		CheckInput ; Input Check is available

EXPORT  	__main	; Make available

__main

BL		InitGPIO; GPIO initialized

MOV		R3,#0x00000001;

MOV		R6,#0;

BL		SignalSend;

Begin		BL		CheckInput;

CMP 	R2,#0

BEQ		Begin;

CMP		R2,#1

BEQ		CW

CMP		R2,#2

BEQ		CCW

B		Begin

CW			AND		R3,R3,#15;

CMP		R3,#1

MOVEQ	R3,#8;

LSRNE	R3,R3,#1;

MOV		R6,R3;

BL		SignalSend;

B		Begin;

CCW			AND		R3,R3,#15;

CMP		R3,#8

MOVEQ	R3,#1;

LSLNE	R3,R3,#1;

MOV		R6,R3;

BL		SignalSend;

B		Begin;

;***************************************************************

; End of the program  section

;***************************************************************

;LABEL      DIRECTIVE       VALUE                           COMMENT

ALIGN

END

Q4)

Q5)

Main code with interrupt changes are provided and modified InputCheck subroutine is provided also.

ISR Code

NVIC_ST_RELOAD 	EQU 0xE000E014

;***************************************************************

; Program section

;***************************************************************

;LABEL		DIRECTIVE	VALUE		COMMENT

;LABEL DIRECTIVE VALUE COMMENT

AREA rutins , CODE, READONLY

;THUMB

EXTERN SignalSend;

EXPORT DriveMotor ;

DriveMotor	PROC

MOV		R4,#2;

CMP		R2,#0;

BX		LR;

CMP		R2,#1

BEQ		CW

CMP		R2,#2

BEQ		CCW

CMP		R2,#3

BEQ		SpeedUp

CMP		R2,#4

BEQ		SpeedDown

CW			AND		R3,R3,#15;

CMP		R3,#1

MOVEQ	R3,#8;

LSRNE	R3,R3,#1;

MOV		R6,R3;

PUSH	{LR}

BL		SignalSend;

POP		{LR}

BX		LR; end

CCW			AND		R3,R3,#15;

CMP		R3,#8

MOVEQ	R3,#1;

LSLNE	R3,R3,#1;

MOV		R6,R3;

PUSH	{LR}

BL		SignalSend;

POP		{LR}

BX		LR; end

SpeedUp		LDR 	R1,=NVIC_ST_RELOAD

LDR		R0,[R1]

UDIV 	R0,R0,R4 ;Interrupt period is decreased

STR		R0,[R1]

MOV		R2,R7;

BX 		LR; end

SpeedDown	LDR 	R1,=NVIC_ST_RELOAD

LDR		R0,[R1]

MUL		R0,R0,R4 ;Interrupt period is increased

STR		R0,[R1]

MOV		R2,R7;

BX 		LR; end

;***************************************************************

; End of the program  section

;***************************************************************

;LABEL      DIRECTIVE       VALUE                           COMMENT

ENDP

END

StartUp.s Change

IMPORT	DriveMotor

SysTick_Handler PROC

EXPORT  SysTick_Handler           [WEAK]

PUSH {LR}

BL DriveMotor

POP {LR}

BX	LR

ENDP

InputCheck

GPIO_PORTB_DATA 	EQU 0x400053FC

;***************************************************************

; Program section

;***************************************************************

;LABEL		DIRECTIVE	VALUE		COMMENT

;LABEL DIRECTIVE VALUE COMMENT

AREA rutins , CODE, READONLY

THUMB

EXPORT CheckInput ;

EXTERN delay;

CheckInput	PROC

LDR		R1,=GPIO_PORTB_DATA; Data address in R1

MOV		R2,#0; R2 holds the information cw or ccw

Check		LDR 	R0,[R1];Checks for any input

LSR		R0,#4;

LSRS	R0,#1;

BCC		Delay100

LSRS	R0,#1;

BCC		Delay100

LSRS	R0,#1;

BCC		Delay100

LSRS	R0,#1;

BCC		Delay100

BX		LR; End

Delay100	MOV32	R0,#1600000;If any input is detected

PUSH{LR}

BL		delay

POP{LR}

LDR 	R0,[R1];	Check Again

LSR		R0,#4;

LSRS	R0,#1;

MOVCC	R2,#1; 1 is the cw direction PB4 pressed

MOVCC R7,#1;

BCC		Released ; If input is detected again wait for relase

LSRS	R0,#1;

MOVCC R2,#2; 2 is the ccw direction PB5 pressed

MOVCC R2,#2;

BCC		Released

LSRS	R0,#1;

MOVCC	R2,#3; 1 is speed up, PB6 pressed

BCC		Released ; If input is detected again wait for relase

LSRS	R0,#1;

MOVCC	R2,#4; 2 is speed down, PB7 pressed

BCC		Released

BX LR; if no signal

Released	LDR 	R0,[R1];	It checks for if the switch is open again

LSR		R0,#4;

LSRS	R0,#1;

BCC		Released;	If it is not open

LSRS	R0,#1;

BCC		Released;

LSRS	R0,#1;

BCC		Released;	If it is not open

LSRS	R0,#1;

BCC		Released;

BX LR; end

;***************************************************************

; End of the program  section

;***************************************************************

;LABEL      DIRECTIVE       VALUE                           COMMENT

ALIGN

ENDP

END

Main Code

;***************************************************************

; Program_Directives.s

; Copies the table from one location

; to another memory location.

; Directives and Addressing modes are

; explained with this program.

;***************************************************************

;***************************************************************

; EQU Directives

; These directives do not allocate memory

;***************************************************************

;LABEL		DIRECTIVE	VALUE		COMMENT

OFFSET  	EQU     	0x10

FIRST	   	EQU	    	0x20000480

STORE		EQU			0x20000410

;***************************************************************

; Directives - This Data Section is part of the code

; It is in the read only section  so values cannot be changed.

;***************************************************************

;LABEL		DIRECTIVE	VALUE		COMMENT

AREA        sdata, DATA, READONLY

THUMB

CTR1    	DCB     	0x10

MSG     	DCB     	"Copying table..."

DCB			0x0D

DCB			0x04

;***************************************************************

; Program section

;***************************************************************

;LABEL		DIRECTIVE	VALUE		COMMENT

AREA    	main, READONLY, CODE

THUMB

EXTERN		OutStr	; Reference external subroutine

EXTERN		InChar; Serial input Added

EXTERN		SignalSend;	GPIO signal send

EXTERN		InitGPIO; GPIO initialize

EXTERN		delay; Delay is available

EXTERN		CheckInput ; Input Check is available

EXTERN		InitSysTick;

EXPORT  	__main	; Make available

__main

BL		InitGPIO; GPIO initialized

MOV		R3,#0x00000001;

MOV		R6,#0;

MOV		R7,#0;

BL		InitSysTick;

CPSIE	I;

Begin		BL		CheckInput;

B		Begin

;***************************************************************

; End of the program  section

;***************************************************************

;LABEL      DIRECTIVE       VALUE                           COMMENT

ALIGN

END

How I approached is that, my input check subroutine can identify the input type, CW, CCW, SpeedUp, SpeedDown, R2 register gets 1,2,3,4 w.r.t. And this subroutine is called in main subroutine continuously. When an interrupt occurs, it looks the what is last input and it processes.